| Electrical characteristics |
| --- |
|  | Nominal voltage (V); 3.6. |
|  | Capacity at C/3 rate (Ah); 16. |
|  | Specific energy (Wh/kg); 85. |
|  | Energy density (Wh/dm3); 186. |
|  | Specific power (W/kg); 1350. |
|  | Power density (W/dm3); 2900. |

| Mechanical characteristics |
| --- |
|  | Diameter (mm); 47. |
|  | Height (mm); 178. |
|  | Weight (kg); 0.680. |
|  | Typical volume (dm3); 0.31. |

| Operating conditions |  |
| --- | --- |
|  | Operating temperature range (°C) as given by the thermal management system; - 20 /+ 55. |
|  | Transport or storage temperature range (°C); - 40 /+ 65. |
|  | Voltage limits : - In charge (V) - In discharge (V); 3.9 2.1. |