Tufts University

Department of Electrical Engineering 
and Computer Science

NERD GIRLS
Maximum Power Point Tracker

Stephanie Chin

Jeanell Gadson

Katie Nordstrom

Project Advisor: Karen Panetta

Project Consultants: Matthew Heller, Richard Colombo, Michael Quaglia

Senior Design Project 2003

##### Final Report

May 12, 2003

Table of Contents

- Purpose

- Introduction

  - Photovoltaic Cells and Array Research

  - Power Supply Research

  - MPPT Research

- Basic Design

  - Why are we building a MPPT?

  - How does it work?

- Implementation

  - Overall Design Considerations

  - Hardware

    - Components

    - Voltage Control

    - Charging Unit

    - Solar Array Protection

  - Software

    - Menu Structure

    - Algorithm

- Assessment

  - Hardware

  - Software

- Conclusion

  - Future Work

- Appendix

  - Hardware Schematics

    - MPPT Block Diagram

    - Circuitry Schematic

  - Software Flowcharts

    - Menu Structure

    - Algorithm

    - Algorithm Submenu Function

  - Code

    - solargirls.asm

    - Lcd.asm

    - Math.asm

    - p2plsp18.lkr

  - Datasheets

    - PIC Microcontroller

    - DC/DC Converter PT4122A

    - DC/DC Converter TPS6734IP

    - PWM TL598CN

    - Diode 16CTU04S

    - LTC DAC 1451CN8

    - MOSFET IXFX90N20Q

    - MOSFET Driver MAX4420CPA

- References

- Acknowledgements

1. Purpose

The objective of the project was to design a Maximum Power Point Tracker (MPPT) for a solar-powered vehicle.  This component optimized the amount of power obtained from the photovoltaic array and charged the power supply.  The solar car will be constructed by the 2003/2004 Nerd Girls Team and will incorporate the Maximum Power Point Tracker unit into the final design.

2. Introduction

Developed by Professor Karen Panetta, the Tufts University Nerd Girls Project brings together a team of multidisciplinary undergraduate female engineers.  Their mission is to build and race a solar-powered vehicle in Fall 2003 and to use it as an outreach tool to introduce engineering to young students.

2.1  Photovoltaic Cells and Array Research

Photovoltaic cells are devices that absorb sunlight and convert that solar energy into electrical energy.

Solar cells are commonly made of silicon, one of the most abundant elements on Earth. Pure silicon, an actual poor conductor of electricity, has four outer valence electrons that form tetrahedral crystal lattices.

The electron clouds of the crystalline sheets are stressed by adding trace amounts of elements that have three or five outer shell electrons that will enable electrons to move. The nuclei of these elements fit well in the crystal lattice, but with only three outer shell electrons, there are too few electrons to balance out, and "positive holes" float in the electron cloud. With five outer shell electrons, there are too many electrons. The process of adding these impurities on purpose is called "doping." When doped with an element with five electrons, the resulting silicon is called N-type ("n" for negative) because of the prevalence of free electrons. Likewise, when doped with an element of three electrons, the silicon is called P-type. The absence of electrons (the "holes") define P-type. 

The combination of N-type and P-type silicon cause an electrostatic field to form at the junction. At the junction, electrons from the sides mix and form a barrier, making it hard for electrons on the N side to cross to the P side. Eventually equilibrium is reached, and an electric field separates the sides. 

When photons (sunlight) hit a solar cell, its energy frees electron-holes pairs. The electric field will send the free electron to the N side and hole to the P side. This causes further disruption of electrical neutrality, and if an external current path is provided, electrons will flow through the path to their original side (the P side) to unite with holes that the electric field sent there, doing work for us along the way. The electron flow provides the current, and the cell's electric field causes a voltage. With both current and voltage, we have power, which is the product of the two.

Three solar cell types are currently available: monocrystalline, polycrystalline, and thin film, discerned by material, efficiency, and composition.

By wiring solar cells in series, the voltage can be increased; or in parallel, the current. Solar cells are wired together to form a solar panel.  Solar panels can be joined to create a solar array.

## 2.2  Power Supply Research

A battery is a source portable electric power.  A storage battery is a reservoir, which may be used repeatedly for storing energy. Energy is charged and drained from the reservoir in the form of electricity, but it is stored as chemical energy.  The most common storage battery is the lead-acid battery that is widely used in automobiles. They represent about 60% of all batteries sold worldwide and are usually more economical and have a high tolerance for abuse.  Lead-acid batteries are inexpensive, relatively safe and easily recyclable, but have a low energy-to-weight ratio, which is a serious limitation when trying to build lightweight vehicles.

New battery technologies are constantly being explored that can offer better energy-to-weight ratios, lower costs and increased battery life. The nickel-metal-hydride battery has received a great deal of attention as a near future solution. Nickel-metal-hydride batteries offer about twice the energy capacity for the same weight as a current lead-acid battery. Another battery type with an even greater energy density is Lithium ion.

## 2.3  MPPT Research

Figure 1:  I-V Curve

The Maximum Power Point Tracker (MPPT) is needed to optimize the amount of power obtained from the photovoltaic array to the power supply.

The output of a solar module is characterized by a performance curve of voltage versus current, called the I-V curve.  See Figure 1.  The maximum power point of a solar module is the point along the I-V curve that corresponds to the maximum output power possible for the module. This value can be determined by finding the maximum area under the current versus voltage curve.

3. Basic Design

3.1  Why are we building a MPPT?

There are commericially available MPPTs which are typically used for home solutions and buildings.  These are not designed to withstand the harsh, fast-changing environmental conditions of solar car racing.  Design of the customized MPPT will ensure that the system operates as closely to the Maximum Power Point (MPP) while being subjected to the varying lighting and temperature.

3.2  How does it work?

The inputs of the MPPT consisted of the photovoltaic voltage and current outputs.  The adjusted voltage and current output of the MPPT charges the power supply.  See Figure 2.

A microcontroller was utilized to regulate the integrated circuits (ICs) and calculate the maximum power point, given the output from the solar array. Hardware and software integration was necessary for the completion of this component.

Figure 2:  Basic Block Diagram

4. Implementation

4.1  Overall Design Considerations

Many factors influenced the component selection and the design of the MPPT.

- In terms of optimal functionality, the theory of power conservation needed to be applied.  The input and output voltage and current were calculated such that the power into and out of the MPPT was equal.

- To protect the photovoltaic array from damage, protection diodes were employed.

- Two 48V lead acid battery banks were utilized.   Only one battery bank will be charged at a time.  (The other will be employed to run other components of the car).

- In order to trickle charge the batteries, a voltage exceeding 48V must be fed to the bank.  In this design, 50V was chosen to charge the power supply.

- To prevent damage and overcharging of the power supply, a FET was employed.

4.2  Hardware

The MPPT circuitry consisted of three sections – Voltage Control, Charging Unit, and Solar Array Protection.  See Appendix 7.1.1.  The Voltage Control block consisted of two DC to DC converters that stepped down the solar array voltage.  The converters supplied the necessary voltage to run the various components of the system.  Secondly, the Charging Unit consisted of the PIC microcontroller, PWM, MOSFET, and protection diodes.  It computed the maximum power point and regulated the various integrated circuits that charged the 48V power supply.  Lastly, the Solar Array Protection block consisted of the protection diodes used to prevent solar panel damage.

4.2.1  Components

Table 1 shows the components used for each of the three sections of the hardware design.  See Appendix 7.4 for datasheets.

| COMPONENT | PART NUMBER |
| --- | --- |
| PIC Microcontroller | PICF458 |
| DC to DC Converter (5V) | PT4122A |
| DC to DC Converter (12V) | TPS6734IP |
| Pulse Width Modulator (PWM) | TL598CN |
| Diode | 16CTU04S |
| Digital to Analog Converter (DAC) | LTC1451CN8 |
| MOSFET | IXFX90N20Q |
| MOSFET driver | MAX4420CPA |

Table 1:  Components

    - Voltage Control

The DC/DC Buck Converter stepped down the solar array output voltage (approximately 48V) to 5v in order to power the PIC, DACs, and RS-232.  The DC/DC Boost Converter stepped up the 5v output from the Buck Converter to 12v in order to power the PWM.

4.2.3  Charging Unit

The charging unit consisted of multiple components, which worked together to power the battery array. This unit contained the ADCs, DACs, PIC microcontroller, PWM, MOSFET, MOSFET driver, inductor, and protection diodes.

The ADC changed the analog output of the solar array into a digital signal to be manipulated by the PIC microcontroller. The DAC worked in the opposite direction of the ADC. It changed the digital output from the PIC to an analog signal, which regulated the PWM.

The PIC microcontroller performed all of the calculations necessary to obtain the maximum power point. The PIC received the input voltage directly from the solar array and converted the value to a digital signal via the ADCs. In order to determine the input current, the output voltage of the voltage divider was sent to the PIC as a digital signal via the ADCs.  From there, knowing the resistance of the voltage divider, the calculations were performed within the PIC. Having both the input voltage (V) and current (I) from the solar array, the power could be determined (P=V*I). Keeping the theory of power conservation in mind, the output power from the PIC needed to equal the input power from the solar array. At the same time, the charging voltage must exceed the battery array voltage, 48V; therefore 50V was assumed for the output voltage. The output current was calculated using the input power and the output voltage. This value was then converted to an analog signal via the DACs and sent to the PWM.

The PWM received the adjusted voltage and current from the PIC, and changed its duty cycle accordingly. This duty cycle controlled the MOSFET.

The MOSFET acted like a switch. When it was on, it closed the circuit and sent the power to ground, preventing the overcharging of the battery array. At this time, current built up in the inductor and it was able to charge.  When it was off, the circuit opened, and the power was sent through the protection diodes to the battery array.  At this time, the inductor discharged.

The protection diodes prevented current from flowing back from the batteries and potentially damaging the solar array.  By placing the diodes in parallel, the overall resistance decreased, and allowed a greater amount of current to pass through.

4.2.4   Solar Array Protection Block

The voltage divider took the voltage from the solar array and stepped it down to a maximum voltage of 4.08V. This prevented the ADC from “blowing out.” Without the voltage divider, the solar array would send too large of a voltage for the ADC to handle. Protection diodes were utilized to prevent the current from flowing back to the solar array and causing damage to it.

Figure 4: MPPT Circuit Board

## 4.3  Software

The PIC Microcontroller chosen had sufficient memory to meet the demands of the design.  The ADCs were also included in the PIC, which reduced the amount of additional external parts.

Programming was completed in MPASM Assembly.  See Appendix 7.2 and 7.3 for Software flowcharts and code.

4.3.1  Menu Structure

The PIC contains a LCD screen, which enabled us to display the input and output voltages and currents.  This enabled us to confirm the results of the calculations performed by the PIC.  The structure of the LCD output was laid out as a menu.  There were four main menu items, Voltage input from the solar array, current input from the solar array, voltage output from the MPPT and current output from the MPPT. See Figure 5.

Initially, the welcoming note was displayed on the LCD followed by the voltage input from the solar array menu item.  A register called which_menu was used to organize the information about which menu item the user was viewing.  Bit 0 of the which_menu register indicated whether or not the user was within the first menu item.  If the bit value was 1, this meant the user was looking at the input voltage from the solar array.  A 0 bit value meant the user was not within this menu item.  The same system was set up for the rest of the menu items.  Bit 1 was allocated to the input current from the solar array menu item.  Bit 2 was allocated to the output voltage from the MPPT menu item.  Finally, bit 3 was allocated to the output current from the MPPT menu item.

By pressing RA4, the user could scroll through the main menu items.  By pushing RB0, the user could view the submenu of each main menu item.  For example, if the user wanted to see the changing input voltage values, the user would scroll through the menu (using the RA4 button) until the Vin Solar menu item was displayed.  Then, the user would select this (pushing RB0) and the voltage would be displayed on the LCD.  The user could return to the main menu by pushing RB0 again.  The which_menu register bit values were used to determine the return location on the main menu.

The final design was set up to perform the calculations to determine the output power each time the user selected the output current from the MPPT menu item.  In order to test the functionality of the calculation code, values were hard-coded for the input voltage, input current and output voltage.  For example, if the voltage input was 5V and the current input was 10mA, the two values were multiplied together to determine the power.  If we wanted a 2V output, this value would be hard-coded as the output voltage.  The input power would be divided by the 2V and the result would be the output current.  So, in this example, the output current would be displayed as 25mA.  This way the power output from the MPPT remained the same as the power input from the solar panels, but the voltage and current were adjusted so that enough voltage would be sent to a power supply to charge it.  See Appendix 7.2.1.

Figure 5:  PIC Microcontroller LCD Menu Display

The topmost figure shows the welcome screen.  The left screens are the scrollable main menus that display a submenu containing input/output data if RBO is selected.  Sample inputs were used to test the calculation algorithm, as shown.

4.3.2  Algorithm

When the program started running, the first steps taken were to configure the PIC ports being used for inputs and outputs and to set the A/D conversion information.  See Appendix 7.2.2.  From there, the output voltage was given a set value.  This value should be 50V, as this was the amount of voltage needed to charge the 48V battery array.

The welcome note was then displayed to inform the user that the program was running.  Following this, the first item on the main menu was displayed (Vin Solar).  At this point the user had the option to either select the item using the RB0 button (and the value would be displayed on the LCD) or to scroll through the four menu items using the RA4 button.

When the user selected one of the menu items by pressing RB0, the program first cleared the which_menu bit that was previously 1 (indicating the last menu item that was viewed).  See Appendix 7.2.3.  The label was then displayed on the LCD screen and the which_menu bit allocated to the current menu item was set to 1.

The program then took the data and either converted the value to a digital signal (if the data was received from port A) and stored the value in a register, or just stored the hard-coded value in a register.   This was the only information needed to display the values for the first three menu items.

If the user selected the current output of the MPPT menu item, the output current was calculated using the input voltage, input current and output voltage values stored in the registers.  The result was then printed to the LCD screen.

In order to return to the correct menu item, the program checked the bit values of the which_menu.  For example, if bit 0 of which_menu was equal to the value of 1, the program would return to the first menu item, Vin Solar.

5. Assessment

5.1  Hardware

DIP packaging was used because they are easier to wire wrap. Wire wrapping for a majority of the circuitry was chosen instead soldering because it will facilitate future changes.

Chip sockets were used instead of wire wrapping directly to the chip; thus if the chip goes bad, it can be replaced and the does not have to be rewired.

The voltage divider circuitry was determined by assuming that the maximum output voltage of the solar array is 75V, and the maximum input of the ADC is 5 volts. See Figure 6.  The following resistor values were used in order to obtain a maximum output of 4.08V:  R1=620K, R2=68K, RL=75K

Figure 6: Voltage Divider Circuitry

Extra diodes were not needed for the Solar Protection Array. Diode protection to VDD and VSS were included in the ADCs on the PIC microcontroller.

The capacitors used do not support high voltages for an extended period of time, therefore they will have a short lifespan.

The packaging for the MOSFET and diodes made it difficult to attach to the circuit board.

The circuitry was placed on multiple boards. This made it easier to visualize the layout, but greatly increased the overall size of the complete device. If the final the device was packaged, the wiring and chips would be protected from damage. Also, the input and output wires would be easily accessible.

Figure 4: PIC Microcontroller

5.2  Software

The calculation section of the program worked with only a few flaws.  We were able to calculate the input power and then determine the output current knowing the output voltage desired and the input power.  However, the code produced incorrect results once the test values were increased to numbers large enough to produce results greater than 256.   The multiplication function was set up to multiply an 8-bit number by another 8-bit number and the result would be 16 bits total, stored in two 8-bit registers.  When the two numbers being multiplied produced a result greater than 256, the value stored in the high bit register was incorrect.  At the same time, we came across problems when the result of the division function included a fraction.  The code was set up to print three decimal values to the LCD (up to 256).  Several different steps were taken in an attempt to print out correct results with fractions; however, the goal was never achieved.

The design was set up so that the PIC would receive an input voltage and current from the solar array.  However, there were difficulties when it came to reading the input values.  Knowing port A was the port used for A/D conversions, it was set up so that there could be two inputs for voltage and current.  There were two registers used to configure the A/D conversion information, ADCON0 and ADCON1.  ADCON0 bit 0 was set to enable the A/D conversion and bits 3-5 were used to determine the channel from which the PIC was reading the input to convert.  Eventually, it should be set up so that bits 5-3 are switched between 000 and 001, taking turns reading the input from channel 0 and channel 1.  In order to test this, however, the bits were hard-coded to 000.  ADCON1 bits 3-0 were set for two inputs (1101).  With two inputs, there needed to be voltage references to ground and +5V.  Ideally, with this test, an input between 0 and 5 volts would be used as the voltage input from the solar array (smaller test values at first).  However, the program constantly shutdown when this design was attempted.

In order to show how the A/D conversion would work, though, the potentiometer values were used as the voltage input.  The potentiometer was defaulted with a link to channel 0 of port A and it seemed that this was the only way to test the A/D conversions.  It was set to convert numbers 0 through 15.  So, in the final design, the user could rotate the knob of the potentiometer to test different values (from 0 to 15) that acted as the input voltage.

Overall, the program was able to meet the requirements of the design, but only to a certain degree.  The final integration of the hardware and software was unable to work due to the troubles encountered when attempting to input or output a voltage to or from the PIC.  The A/D conversion and the calculations could be tested with the final program however.  The finished program consisted of a hard-coded value of 4mA for the input current and 2V for the output voltage.  The user could test the program by rotating the potentiometer value (acting as the input voltage) and the result could be viewed under the Iout MPPT menu item.  For example, the user could turn the potentiometer so that the value of the input voltage was 5V.  The program would calculate the power using this and the 4mA hard-coded.  The output current would then be determined using this power value and the output voltage of 2V.  The result in this case would be 10mA.

6. Conclusion

In order to charge a power source at its maximum efficiency, a Maximum Power Point Tracker (MPPT) device is utilized.  The MPPT design incorporated three systems - the Voltage Divider, Charging Unit, and Solar Array Protection.

Although the final MPPT did not completely function as planned, the software algorithm did complete the correct calculation to find the Maximum Power Point.  As the project came to an end, various changes could have been made which could benefit the design and implementation process.  A smaller output range of the solar array would have helped to design a more efficient MPPT.  Allowance of ample time is necessary.  Many problems with the component purchasing and software were encountered.

There were a few weaknesses in the code.  First, the PIC was not programmed to continuously loop.  A program that automatically checks and updates the maximum power point could improve the design.    Secondly, the program did not successfully communicate with the hardware.  Working communication is absolutely crucial in the final device that will be incorporated into the solar-powered vehicle.

Use of space in the car is also an important factor, as it can be critical to the overall design.  A more organized circuitry layout on only one board would enable the device to be simply set into the car.

6.1  Future Work

Fast-switching components are necessary to operate the device intended for solar car racing.  The component choice is key in the design of the MPPT.  High power efficiency is attained by carefully researching and selected the right components.

7. Appendix

Contents

  - Hardware Schematics

    - MPPT Block Diagram

    - Circuitry Schematic

  - Software Flowcharts

    - Menu Structure

    - Algorithm

    - Algorithm Submenu Function

  - Code

    - solargirls.asm

    - Lcd.asm

    - Math.asm

    - p2plsp18.lkr

  - Datasheets

    - PIC Microcontroller

    - DC/DC Converter PT4122A

    - DC/DC Converter TPS6734IP

    - PWM TL598CN

    - Diode 16CTU04S

    - LTC DAC 1451CN8

    - MOSFET IXFX90N20Q

    - MOSFET Driver MAX4420CPA

Appendix 7.1  Hardware Schematics

Appendix 7.1.1  MPPT Block Diagram

Appendix 7.1.2  Circuitry Schematic

Appendix 7.2  Software Flowcharts

Appendix 7.2.1  Menu Structure

Appendix 7.2.2  Algorithm

Appendix 7.2.3  Algorithm Submenu Function

Appendix 7.3  Code

Appendix 7.3.1  solargirls.asm

;************************************************************************

;*	Microchip Technology Inc. 2002

;*	Assembler version: 2.0000

;*	Filename:

;*		solargirls.asm (main routine)

;*	Dependents:

;*		p18lcd.asm

;*		p18math.asm

;*		16f877.lkr

;************************************************************************

;MAXIMUM POWER POINT TRACKER PIC CODE

;STEPHANIE, KATIE, JEANELL

;************************************************************************

list p=18f452

#include p18f452.inc

;Program Configuration Registers

__CONFIG    _CONFIG2L, _BOR_OFF_2L & _PWRT_ON_2L

__CONFIG    _CONFIG4L, _STVR_OFF_4L & _LVP_OFF_4L & _DEBUG_OFF_4L

__CONFIG    _CONFIG5L, _CP0_OFF_5L & _CP1_OFF_5L & _CP2_OFF_5L & _CP3_OFF_5L

__CONFIG    _CONFIG6L, _WRT0_OFF_6L & _WRT1_OFF_6L & _WRT2_OFF_6L & _WRT3_OFF_6L

__CONFIG    _CONFIG7L, _EBTR0_OFF_7L & _EBTR1_OFF_7L & _EBTR2_OFF_7L & _EBTR3_OFF_7L

#define	scroll_dir	TRISA,4

#define	scroll		PORTA,4		;Push-button RA4 on PCB

#define	select_dir	TRISB,0

#define	select		PORTB,0		;Push-button RB0 on PCB

EXTERN	LCDInit, temp_wr, d_write, i_write, LCDLine_1, LCDLine_2

EXTERN	UMUL0808L, UDIV1608L, AARGB0, AARGB1, BARGB0, BARGB1, AARGB5, REMB0, REMB1, TEMP

ssprw	macro				;check for idle SSP module routine

movlw	0x00

andwf	SSPCON2,W

sublw	0x00

btfss	STATUS,Z

bra	$-8

btfsc	SSPSTAT,R_W

bra	$-2

endm

variables	UDATA

which_menu  RES 1

ptr_pos		RES 1

ptr_count	RES 1

temp_1		RES 1

temp_2		RES 1

temp_3		RES 1

cmd_byte	RES 1

temperature	RES 1

LSD		RES 1

MsD		RES 1

MSD		RES 1

seconds		RES 1

minutes		RES 1

hours		RES 1

NumH		RES 1

NumL		RES 1

TenK		RES 1

Thou		RES 1

Hund		RES 1

Tens		RES 1

Ones		RES 1

volt_in      	RES 1

curr_in			RES 1

batt_volt		RES 1

batt_curr		RES 1

STARTUP CODE

NOP

goto	start

NOP

NOP

NOP

PROG1 	CODE

stan_table				;table for standard code

;	    "XXXXXXXXXXXXXXXX"

;				ptr:

data	"  Vin (Solar)   "	;0

data	"  Iin (Solar)   "	;16

data	"  Vout (MPPT)   "	;32

data	"  Iout (MPPT)   "	;48

data	"RA4=Next RB0=Sel"	;64

data	"   Nerd Girls   "  ;80

data	"   MPPT Rocks   "	;96

data	"RA4=Set RB0=Menu"	;112

data	"RA4= --> RBO= ++"	;128

data	"   RB0 = Exit 	 "	;144

data	"Volts = "	;160

data	"Current =       "	;176

data	"                "	;192

start

call LCDInit

movlw	B'10100100'		;initialize USART

movwf	TXSTA			;8-bit, Async, High Speed

movlw	.25

movwf	SPBRG			;9.6kbaud @ 4MHz

movlw	B'10010000'

movwf	RCSTA

bcf	TRISC,2		;configure CCP1 module for buzzer

;	bcf	TRISC,6

movlw	0x80

movwf	PR2			;initialize PWM period

movlw	0x80			;initialize PWM duty cycle

movwf	CCPR1L

bcf	CCP1CON,CCP1X

bcf	CCP1CON,CCP1Y

movlw	0x05			;postscale 1:1, prescaler 4, Timer2 ON

movwf	T2CON

bsf	TRISA,4		;make switch RA4 an Input

;ADDITIONS FOR A/D CONVERTING

clrf	PORTB			;Clear PORTB

clrf	TRISB			;PORTB all outputs, display 4 MSB's

;of A/D result on LEDs

bsf	TRISB,0		;make switch RB0 an Input

movlw	B'01000001' 	;Fosc/8, A/D enabled

movwf	ADCON0

movlw	B'00001110'		;B'00001110';Left justify,1 analog channel

movwf	ADCON1		;VDD and VSS references

; test register value print by putting a value into curr_in

;	movlw 	B'00000011'	; put value in register W (35)

;	movwf 	volt_in	; put value of reg. W into volt_in

movlw	B'00000100'		;B'01100100'

movwf 	curr_in	; put value of reg. W into curr_in

movlw   B'00000010'	; put value 50 (50v output to batt) in reg. W

movwf   batt_volt		;put value of reg. W (50) into batt_volt reg.

;**************** STANDARD CODE MENU SELECTION *****************

;Introduction

movlw	.80			;send "Nerd Girls" to LCD

movwf	ptr_pos

call	stan_char_1

movlw	.96			;send "MPPT Rocks" to LCD

movwf	ptr_pos

call	stan_char_2

call	delay_1s		;delay for display

call	delay_1s		;delay for display

call	delay_1s		;delay for display

call	delay_1s		;delay for display

menu

;------------------ VOLTAGE IN (SOLAR)----------------------------

bcf which_menu, 3		;bit 3 of register which_menu is cleared to 0

btfss	scroll		;wait for RA4 release

goto	$-2

btfss	select		;wait for RB0 release

goto	$-2

movlw	0x00			;Displays "Solar Vout" (.0) to LCD

movwf	ptr_pos

call	stan_char_1

movlw	.64			;RA4=Next  RB0=Sel

movwf	ptr_pos

call	stan_char_2

v_wait

bsf which_menu, 0

btfss	select		;voltmeter measurement ??

bra	voltmeter

btfsc	scroll		;next mode ??

bra	v_wait	 	;NO

btfss	scroll		;YES

bra	$-2			;wait for RA4 release

;-------------- SOLAR CURRENT OUTPUT -------------------------------

menu_buz

bcf which_menu, 0       	;bit 0 of register which_menu is cleared to 0

btfss	scroll		;wait for RA4 release

goto	$-2

btfss	select		;wait for RB0 release

bra	$-2

movlw	.16			;Displays "Solar Iout" to LCD

movwf	ptr_pos

call	stan_char_1

movlw	.64			;RA4=Next  RB0=Sel

movwf	ptr_pos

call	stan_char_2

b_wait

bsf which_menu, 1

btfss select		;current measurement??

bra voltmeter

btfsc scroll		;next mode??

bra b_wait			;NO

btfss scroll		;YES

bra $-2			;wait for RA4 release

;---------------- MPPT VOLTAGE OUTPUT ----------------------

menu_temp

bcf which_menu, 1		;bit 1 of register which_menu is cleared to 0

btfss	scroll		;wait for RA4 release

bra	$-2

btfss	select		;wait for RB0 release

bra	$-2

movlw	.32			    ;Display "MPPT Vout" to LCD

movwf	ptr_pos

call	stan_char_1

movlw	.64			    ;RA4=Next  RB0=Sel

movwf	ptr_pos

call	stan_char_2

t_wait

bsf which_menu, 2

btfss select			;current measurement??

bra voltmeter

btfsc scroll			;next mode??

bra t_wait				;NO

btfss scroll			;YES

bra $-2				;wait for RA4 release

;------------- -- MPPT CURRENT OUTPUT ----------------------------------

menu_clock

bcf which_menu, 2		;bit 2 of register which_menu is cleared to 0

btfss	scroll		;wait for RA4 release

bra	$-2

btfss	select		;wait for RB0 release

bra	$-2

movlw	.48		  	;Display "MPPT Iout" to LCD

movwf	ptr_pos

call	stan_char_1

movlw	.64			;RA4=Next  RB0=Sel

movwf	ptr_pos

call	stan_char_2

;---------------------don't need clock stuff--------------------

c_wait

bsf which_menu, 3

btfss select			;current measurement??

bra voltmeter

btfsc scroll			;next mode??

bra c_wait				;NO

btfss scroll			;YES

bra $-2					;wait for RA4 release

;	btfss	select			;goto time ??

;	bra	clock			;YES

;	btfsc	scroll			;NO, next mode ??

;	bra	c_wait			;NO

;	btfss	scroll			;YES

;	bra	$-2			;wait for release

;----------------- ---end of clock stuff------------------------

bra	menu			;begining of menu

return

;************* STANDARD USER CODE *****************************

;------------- Voltmeter---------------------------------------

voltmeter

btfss	select			;wait for RB0 release

bra	$-2

;------------------------------ ---------------------------------

;ADDITIONS FOR A/D CONVERTING

;write in 001 for bits 5-3 of adcon0

bsf	ADCON0,GO	;Start A/D conversion (changes bit 2 of ADCON0 to 1)

Wait

btfss	PIR1,ADIF	;Wait for conversion to complete

goto	Wait

swapf	ADRESH,W	;Swap A/D result nibbles

andlw	0x0f		;Mask off lower 4 bits

movwf	volt_in	;Write A/D result to PORTB

;*************************************************************

;perform calculations

btfsc   which_menu, 0	;if selected solar voltage output (bit 0 of reg. which_menu would then be 1)

goto 	temp_inputvoltprint

btfsc   which_menu, 2	;if selected mppt voltage output (bit 2 of reg. which_menu would then be 1)

goto 	temp_outputvoltprint

btfsc   which_menu, 1	;if selected solar current output (bit 1 of reg. which_menu would then be 1)

goto 	temp_inputcurrprint

btfsc   which_menu, 3	;if selected mppt current output (bit 3 of reg. which_menu would then be 1)

goto 	temp_outputcurrprint	;send "Current = " to the LCD

movwf	ptr_pos

call	stan_char_1

temp_inputvoltprint

call 	LCDLine_1

;movlw	0x20		;space

;movwf	temp_wr

;call	d_write

;movlw	0x20		;space

;movwf	temp_wr

;call	d_write

movlw	A'V'			;print "V"

movwf	temp_wr

call	d_write

movlw	A'O'			;print "O"

movwf	temp_wr

call	d_write

movlw	A'L'

movwf	temp_wr

call	d_write

movlw	A'T'

movwf	temp_wr

call	d_write

movlw	A'S'

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movlw	A'I'

movwf	temp_wr

call	d_write

movlw	A'N'

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movlw	A'='			;print "="

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movf 	volt_in, W		;print Digital Input test value

call	bin_bcd		;get temp ready for LCD

movf	MSD,W			;send high digit

movwf	temp_wr

call	d_write

movf	MsD,W			;send middle digit

movwf	temp_wr

call	d_write

movf	LSD,W			;send low digit

movwf	temp_wr

call	d_write

movlw	A'V'

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

;end of sending unit to LCD

movwf	temp_wr

call	d_write

goto 	volts_again

;-------------------------------------------

temp_inputcurrprint

call 	LCDLine_1

;movlw	0x20		;space

;movwf	temp_wr

;call	d_write

;movlw	0x20		;space

;movwf	temp_wr

;call	d_write

movlw	A'C'			;print "C"

movwf	temp_wr

call	d_write

movlw	A'U'			;print "U"

movwf	temp_wr

call	d_write

movlw	A'R'

movwf	temp_wr

call	d_write

movlw	A'R'

movwf	temp_wr

call	d_write

;movlw	A'E'

;movwf	temp_wr

;call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movlw	A'I'

movwf	temp_wr

call	d_write

movlw	A'N'

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movlw	A'='			;print "="

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movf 	curr_in, W	    	;print Digital Input test value

call	bin_bcd			;get temp ready for LCD

movf	MSD,W			;send high digit

movwf	temp_wr

call	d_write

movf	MsD,W			;send middle digit

movwf	temp_wr

call	d_write

movf	LSD,W			;send low digit

movwf	temp_wr

call	d_write

movlw	A'm'

movwf	temp_wr

call	d_write

movlw	A'A'

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

;end of sending unit to LCD

movwf	temp_wr

call	d_write

goto 	volts_again

;------------------------------------

;-------------------------------------------

temp_outputvoltprint

call 	LCDLine_1

;movlw	0x20			;space

;movwf	temp_wr

;call	d_write

;movlw	0x20			;space

;movwf	temp_wr

;call	d_write

movlw	A'V'				;print "C"

movwf	temp_wr

call	d_write

movlw	A'O'				;print "U"

movwf	temp_wr

call	d_write

movlw	A'L'

movwf	temp_wr

call	d_write

movlw	A'T'

movwf	temp_wr

call	d_write

movlw	A'S'

movwf	temp_wr

call	d_write

movlw	0x20				;space

movwf	temp_wr

call	d_write

movlw	A'O'

movwf	temp_wr

call	d_write

movlw	A'U'

movwf	temp_wr

call	d_write

movlw	A'T'

movwf	temp_wr

call	d_write

movlw	0x20				;space

movwf	temp_wr

call	d_write

movlw	A'='				;print "="

movwf	temp_wr

call	d_write

movlw	0x20				;space

movwf	temp_wr

call	d_write

movf 	batt_volt, W	   	;print Digital Input test value

call	bin_bcd			;get temp ready for LCD

movf	MSD,W			;send high digit

movwf	temp_wr

call	d_write

movf	MsD,W			;send middle digit

movwf	temp_wr

call	d_write

movf	LSD,W			;send low digit

movwf	temp_wr

call	d_write

movlw	A'V'

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

;end of sending unit to LCD

movwf	temp_wr

call	d_write

goto 	volts_again

;------------------------------------

;-------------------------------------------

temp_outputcurrprint

call 	LCDLine_1

;movlw	0x20			;space

;movwf	temp_wr

;call	d_write

;movlw	0x20			;space

;movwf	temp_wr

;call	d_write

movlw	A'C'			;print "C"

movwf	temp_wr

call	d_write

movlw	A'U'			;print "U"

movwf	temp_wr

call	d_write

movlw	A'R'

movwf	temp_wr

call	d_write

movlw	A'R'

movwf	temp_wr

call	d_write

;movlw	A'E'

;movwf	temp_wr

;call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movlw	A'O'

movwf	temp_wr

call	d_write

movlw	A'U'

movwf	temp_wr

call	d_write

movlw	A'T'

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movlw	A'='			;print "="

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

movf	volt_in, W

movwf   AARGB0			;voltage in

movf	curr_in, W

movwf	BARGB0		;current in

call	UMUL0808L		;multiply BARGB0 by AARGB0 (result stored in BARGB1 (high) and AARGB1 (low)

movf	BARGB1, W		;high bit register result of mult stored in BARGB0

movwf	BARGB0

movf	AARGB1, W		;low bit register result of mult stored in AARGB0

movwf	AARGB0

movf	batt_volt, W		;store 50V (volt. output to batteries) in BARGB1

movwf	BARGB1

call    UDIV1608L			;[BARGB0][AARGB0]/[BARGB1] result stored in AARGB1

movf	AARGB1, W		;storing result in AARGB1 and sending to reg. W to print

call 	bin_bcd

movf	MSD,W

movwf   temp_wr

call 	d_write

movf    MsD,W

movwf   temp_wr

call 	d_write

movf	LSD,W			;send high digit from the LSD #.xx

movwf	temp_wr

call	d_write

;	movf	volt_in, W		;moves input voltage into reg. W

;	mulwf	curr_in			;multiplies the input volt. and input curr, stores result in W

;	movf 	PRODL, W

;	movwf   AARGB0

;	movf    PRODH, W

;	movwf	AARGB1

;	movf	batt_volt, W            ;6

;	movwf	BARGB0

;	call 	UDIV1608L

;movf	AARGB0, W		;prepare for 16-bit binary to BCD

;movwf	NumH

;movf	AARGB1, W

;movwf	NumL

;call	bin16_bcd		;get volts ready for LCD

;	call	LCDLine_2		;display A/D result on 2nd line

;movf	Hund,W			;get hunds

;call	bin_bcd

;movf    MsD,W

;movwf   temp_wr

;call 	d_write

;movf	LSD,W			;send high digit from the LSD #.xx

;movwf	temp_wr

;call	d_write

;	movf	AARGB1, W

;	call    bin_bcd

;	movf	MSD,W			;send high digit

;	movwf	temp_wr

;	call	d_write

;	movf	MsD,W			;send middle digit

;	movwf	temp_wr

;	call	d_write

;	movf	LSD,W			;send low digit

;	movwf	temp_wr

;	call	d_write

movlw	A'm'

movwf	temp_wr

call	d_write

movlw	A'A'

movwf	temp_wr

call	d_write

movlw	0x20			;space

movwf	temp_wr

call	d_write

;end of sending unit to LCD

movwf	temp_wr

call	d_write

goto 	volts_again

;------------------------------------

volts_again

movlw	.144			;Display "RB0 = Exit" to LCD

movwf	ptr_pos

call	stan_char_2

movlw	"\r"			;move data into TXREG

movwf	TXREG			;carriage return

btfss	TXSTA,TRMT		;wait for data TX

bra	$-2

btfsc	select	;exit volt measurement ?? - if register select is 0, then skip next instruction and exit

bra	    voltmeter	;NO, do conversion again

btfsc   which_menu, 0	;YES, if bit 0 of register which_menu is 0, then skip next instruction

bra	    menu	   		; branches to next menu item (solar current output)

btfsc   which_menu, 1	;YES, if bit 1 of reg. which_menu is 0, skips next instr

bra     menu_buz		; branches to next menu item (mppt voltage output)

btfsc   which_menu, 2	;YES, if bit 2 of reg. which_menu is 0, skips next instr

bra     menu_temp 		; branches to next menu item (mppt current output)

btfsc   which_menu, 3	;YES, if bit 3 of reg. which_menu is 0, skips next instr

bra     menu_clock

;----------------- CLOCK ------------------------------------------

clock

btfss	select		;wait for RB0 button release

bra	$-2

movlw	0x0F			;intitialize TIMER1

movwf	T1CON

clrf	seconds

clrf	minutes

clrf	hours

overflow

bcf	PIR1,TMR1IF

movlw	0x80

movwf	TMR1H			;load regs for 1 sec overflow

clrf	TMR1L

incf	seconds,F		;increment seconds

movf	seconds,W

sublw	.60

btfss	STATUS,Z		;increment minutes ?

bra	clk_done

incf	minutes,F

clrf	seconds

movf	minutes,W

sublw	.60

btfss	STATUS,Z		;increment hours ?

bra	clk_done

incf	hours,F

clrf	minutes

movf	hours,W

sublw	.13

btfss	STATUS,Z

bra	clk_done

movlw	.1			;start a new 12 hour period

movwf	hours

clk_done

movf	hours,W			;send hours to LCD

call	bin_bcd

call	LCDLine_1		;place time on line 1

movf	MsD,W			;send middle digit

movwf	temp_wr

call	d_write

movf	LSD,W			;send low digit

movwf	temp_wr

call	d_write

movlw	0x3A			;send  :   colon

movwf	temp_wr

call	d_write

movf	minutes,W		;send minutes to LCD

call	bin_bcd

movf	MsD,W			;send middle digit

movwf	temp_wr

call	d_write

movf	LSD,W			;send low digit

movwf	temp_wr

call	d_write

movlw	0x3A			; send :   colon

movwf	temp_wr

call	d_write

movf	seconds,W		;send seconds to LCD

call	bin_bcd

movf	MsD,W			;send middle digit

movwf	temp_wr

call	d_write

movf	LSD,W			;send low digit

movwf	temp_wr

call	d_write

movlw	0x20			;send 3 spaces after 00:00:00

movwf	temp_wr

call	d_write

movlw	0x20

movwf	temp_wr

call	d_write

movlw	0x20

movwf	temp_wr

call	d_write

movlw	.112			;send "RA4=Dn RB0=Menu" to LCD

movwf	ptr_pos

call	stan_char_2

btfss	scroll		;set time ??

bra	set_time

btfss	select		;return to main menu ??

bra	menu

btfss	PIR1,TMR1IF		;has timer1 overflowed ?

bra	$-2			;NO, wait til overflow

bra	overflow		;YES

return

;***************************** *******************************

;************************** ROUTINES *************************

;*************************************************************

;*************************************************************

;----Standard code, Place characters on line-1-----------------------

stan_char_1

call	LCDLine_1		;move cursor to line 1

movlw	.16			;1-full line of LCD

movwf	ptr_count

movlw	UPPER stan_table

movwf	TBLPTRU

movlw	HIGH stan_table

movwf	TBLPTRH

movlw	LOW stan_table

movwf	TBLPTRL

movf	ptr_pos,W

addwf	TBLPTRL,F

clrf	WREG

addwfc	TBLPTRH,F

addwfc	TBLPTRU,F

stan_next_char_1

tblrd	*+

movff	TABLAT,temp_wr

call	d_write			;send character to LCD

decfsz	ptr_count,F		;move pointer to next char

bra	stan_next_char_1

movlw	"\n"			;move data into TXREG

movwf	TXREG			;next line

btfss	TXSTA,TRMT		;wait for data TX

goto	$-2

movlw	"\r"			;move data into TXREG

movwf	TXREG			;carriage return

btfss	TXSTA,TRMT		;wait for data TX

goto	$-2

return

;----Standard code, Place characters on line-2----------------------

stan_char_2

call	LCDLine_2		;move cursor to line 2

movlw	.16			;1-full line of LCD

movwf	ptr_count

movlw	UPPER stan_table

movwf	TBLPTRU

movlw	HIGH stan_table

movwf	TBLPTRH

movlw	LOW stan_table

movwf	TBLPTRL

movf	ptr_pos,W

addwf	TBLPTRL,F

clrf	WREG

addwfc	TBLPTRH,F

addwfc	TBLPTRU,F

stan_next_char_2

tblrd	*+

movff	TABLAT,temp_wr

call	d_write			;send character to LCD

decfsz	ptr_count,F		;move pointer to next char

bra	stan_next_char_2

movlw	"\n"			;move data into TXREG

movwf	TXREG			;next line

btfss	TXSTA,TRMT		;wait for data TX

goto	$-2

movlw	"\r"			;move data into TXREG

movwf	TXREG			;carriage return

btfss	TXSTA,TRMT		;wait for data TX

goto	$-2

return

;--------------------------------------------- --------------------

;------------------ 100ms Delay -----------------------------

delay_100ms

movlw	0xFF

movwf	temp_1

movlw	0x83

movwf	temp_2

d100l1

decfsz	temp_1,F

bra	d100l1

decfsz	temp_2,F

bra	d100l1

return

;---------------- 1s Delay --------------------------------

delay_1s

movlw	0xFF

movwf	temp_1

movwf	temp_2

movlw	0x05

movwf	temp_3

d1l1

decfsz	temp_1,F

bra	d1l1

decfsz	temp_2,F

bra	d1l1

decfsz	temp_3,F

bra	d1l1

return

;---------------- Set Current Time -------------------------

set_time

movlw	.128			;send "RA4= --> RBO= ++" to LCD

movwf	ptr_pos

call	stan_char_2

set_time_again

btfss	scroll		;wait for button release

bra	$-2

call	LCDLine_1		;start at 0x00 on LCD

btfss	select		;wait for RB0 button release

bra	$-2

call	delay_100ms

btfss  	select	;increment hours (tens) ?

bra	inc_hours

bra	next_digit

inc_hours

incf	hours

movf	hours,W		;check if hours has passed 12 ?

sublw	.13

btfss	STATUS,Z

bra	next_digit

clrf	hours			;YES, reset hours to 00

next_digit

btfss	scroll		;move to next digit

bra	inc_mins

movf	hours,W

call	bin_bcd		;get hours ready for display

movf	MsD,W			;send tens digit

movwf	temp_wr

call	d_write

movf	LSD,W			;send ones digit

movwf	temp_wr

call	d_write

movlw	0x3A			;send   :   colon

movwf	temp_wr

call	d_write

bra	set_time_again

inc_mins

btfss	scroll		;wait for RA4 button release

bra	$-2

call	LCDLine_1

movlw	0x14			;shift cursor to right 3 places

movwf	temp_wr

call	i_write

movlw	0x14

movwf	temp_wr

call	i_write

movlw	0x14

movwf	temp_wr

call	i_write

btfss	select		;wait for RB0 button release

bra	$-2

call	delay_100ms

btfss  	select	;increment minutes (tens) ?

bra	inc_minutes

bra	next_digit?

inc_minutes

incf	minutes

movf	minutes,W		;check if hours has passed 12 ?

sublw	.60

btfss	STATUS,Z

bra	next_digit?

clrf	minutes

next_digit?

btfss	scroll			;move to next digit

bra	set_time_done

movf	minutes,W

call	bin_bcd			;get minutes ready for display

movf	MsD,W			;send tens digit

movwf	temp_wr

call	d_write

movf	LSD,W			;send ones digit

movwf	temp_wr

call	d_write

movlw	0x3A			;send  :   colon

movwf	temp_wr

call	d_write

bra	inc_mins

set_time_done

btfss	scroll		;wait for RA4 button release

bra	$-2

bra	overflow

;---------------- Binary (8-bit) to BCD ------------------

;		255 = highest possible result

bin_bcd

clrf	MSD

clrf	MsD

movwf	LSD		;move value to LSD

ghundreth

movlw	.100		;subtract 100 from LSD

subwf	LSD,W

btfss	STATUS,C	;is value greater than 100

bra	gtenth		;NO goto tenths

movwf	LSD		;YES, move subtraction result into LSD

incf	MSD,F		;increment hundreths

bra	ghundreth

gtenth

movlw	.10		;take care of tenths

subwf	LSD,W

btfss	STATUS,C

bra	over		;finished conversion

movwf	LSD

incf	MsD,F		;increment tenths position

bra	gtenth

over				;0 - 9, high nibble = 3 for LCD

movf	MSD,W		;get BCD values ready for LCD display

xorlw	0x30		;convert to LCD digit

movwf	MSD

movf	MsD,W

xorlw	0x30		;convert to LCD digit

movwf	MsD

movf	LSD,W

xorlw	0x30		;convert to LCD digit

movwf	LSD

retlw	0

;---------------- Binary (16-bit) to BCD -----------------------

;		xxx = highest possible result

bin16_bcd

; Takes number in NumH:NumL

; Returns decimal in

; TenK:Thou:Hund:Tens:Ones

swapf   NumH,W

andlw   0x0F

addlw   0xF0

movwf   Thou

addwf   Thou,F

addlw   0xE2

movwf   Hund

addlw   0x32

movwf   Ones

movf    NumH,W

andlw   0x0F

addwf   Hund,F

addwf   Hund,F

addwf   Ones,F

addlw   0xE9

movwf   Tens

addwf   Tens,F

addwf   Tens,F

swapf   NumL,W

andlw   0x0F

addwf   Tens,F

addwf   Ones,F

rlcf     Tens,F

rlcf     Ones,F

comf    Ones,F

rlcf     Ones,F

movf    NumL,W

andlw   0x0F

addwf   Ones,F

rlcf     Thou,F

movlw   0x07

movwf   TenK

movlw   0x0A                     ; Ten

Lb1:

decf    Tens,F

addwf   Ones,F

btfss   STATUS,C

bra   Lb1

Lb2:

decf    Hund,F

addwf   Tens,F

btfss   STATUS,C

bra   Lb2

Lb3:

decf    Thou,F

addwf   Hund,F

btfss   STATUS,C

bra   Lb3

Lb4:

decf    TenK,F

addwf   Thou,F

btfss   STATUS,C

bra   Lb4

retlw   0

;---------------------------- EEPROM WRITE -----------------------------

write_eeprom

bsf	SSPCON2,SEN		;start bit

btfsc	SSPCON2,SEN

goto	$-2

movlw	B'10100000'		;send control byte (write)

movwf	SSPBUF

ssprw

btfsc	SSPCON2,ACKSTAT		;ack?

goto	$-2

movlw	0x00			;send slave address HIGH byte

movwf	SSPBUF

ssprw

btfsc	SSPCON2,ACKSTAT		;ack?

goto	$-2

movlw	0x05			;send slave address LOW byte(0x0005)

movwf	SSPBUF

ssprw

btfsc	SSPCON2,ACKSTAT		;ack?

goto	$-2

movf	temperature,w		;send slave DATA = temperature

movwf	SSPBUF

ssprw

btfsc	SSPCON2,ACKSTAT		;ack?

goto	$-2

bsf	SSPCON2,PEN		;stop bit

btfsc	SSPCON2,PEN

goto	$-2

bcf	PIR1,TMR1IF		;clear TIMER1 overflow flag

clrf	TMR1L			;clear registers for next overflow

clrf	TMR1H

return

;**************************** ****************************

end

Appendix 7.3.1  Lcd.asm

;********************************************************************

;*	Microchip Technology Inc. 2002

;*	Assembler version: 2.0000

;*	Filename:

;*		p18lcd.asm (main routine)

;*	Dependents:

;*		p18demo.asm

;*		p18math.asm

;*		16f877.lkr

;********************************************************************

list p=18f452

#include p18f452.inc

#define	LCD_D4		PORTD, 0	; LCD data bits

#define	LCD_D5		PORTD, 1

#define	LCD_D6		PORTD, 2

#define	LCD_D7		PORTD, 3

#define	LCD_D4_DIR	TRISD, 0	; LCD data bits

#define	LCD_D5_DIR	TRISD, 1

#define	LCD_D6_DIR	TRISD, 2

#define	LCD_D7_DIR	TRISD, 3

#define	LCD_E		PORTA, 1	; LCD E clock

#define	LCD_RW		PORTA, 2	; LCD read/write line

#define	LCD_RS		PORTA, 3	; LCD register select line

#define	LCD_E_DIR	TRISA, 1

#define	LCD_RW_DIR	TRISA, 2

#define	LCD_RS_DIR	TRISA, 3

#define	LCD_INS		0

#define	LCD_DATA	1

D_LCD_DATA	UDATA

COUNTER		res	1

delay		res	1

temp_wr		res	1

temp_rd		res	1

GLOBAL	temp_wr

PROG1	CODE

;***************************************************************************

LCDLine_1

movlw	0x80

movwf	temp_wr

rcall	i_write

return

GLOBAL	LCDLine_1

LCDLine_2

movlw	0xC0

movwf	temp_wr

rcall	i_write

return

GLOBAL	LCDLine_2

;write data

d_write

movff	temp_wr,TXREG

btfss	TXSTA,TRMT

goto	$-2

rcall	LCDBusy

bsf	STATUS, C

rcall	LCDWrite

return

GLOBAL	d_write

;write instruction

i_write

rcall	LCDBusy

bcf	STATUS, C

rcall	LCDWrite

return

GLOBAL	i_write

rlcd	macro	MYREGISTER

IF MYREGISTER == 1

bsf	STATUS, C

rcall	LCDRead

ELSE

bcf	STATUS, C

rcall	LCDRead

ENDIF

endm

;********************************************************************

; *******************************************************************

LCDInit

clrf	PORTA

bcf	LCD_E_DIR		;configure control lines

bcf	LCD_RW_DIR

bcf	LCD_RS_DIR

movlw	b'00001110'

movwf	ADCON1

movlw	0xff			; Wait ~15ms @ 20 MHz

movwf	COUNTER

lil1

movlw	0xFF

movwf	delay

rcall	DelayXCycles

decfsz	COUNTER,F

bra	lil1

movlw	b'00110000'		;#1 Send control sequence

movwf	temp_wr

bcf	STATUS,C

rcall	LCDWriteNibble

movlw	0xff			;Wait ~4ms @ 20 MHz

movwf	COUNTER

lil2

movlw	0xFF

movwf	delay

rcall	DelayXCycles

decfsz	COUNTER,F

bra	lil2

movlw	b'00110000'		;#2 Send control sequence

movwf	temp_wr

bcf	STATUS,C

rcall	LCDWriteNibble

movlw	0xFF			;Wait ~100us @ 20 MHz

movwf	delay

rcall	DelayXCycles

movlw	b'0011000'		;#3 Send control sequence

movwf	temp_wr

bcf	STATUS,C

rcall	LCDWriteNibble

;test delay

movlw	0xFF			;Wait ~100us @ 20 MHz

movwf	delay

rcall	DelayXCycles

movlw	b'00100000'		;#4 set 4-bit

movwf	temp_wr

bcf	STATUS,C

rcall	LCDWriteNibble

rcall	LCDBusy		;Busy?

movlw	b'00101000'		;#5   Function set

movwf	temp_wr

rcall	i_write

movlw	b'00001101'		;#6  Display = ON

movwf	temp_wr

rcall	i_write

movlw	b'00000001'		;#7   Display Clear

movwf	temp_wr

rcall	i_write

movlw	b'00000110'		;#8   Entry Mode

movwf	temp_wr

rcall	i_write

movlw	b'10000000'		;DDRAM addresss 0000

movwf	temp_wr

rcall	i_write

return

GLOBAL	LCDInit

; *******************************************************************

;********************************************************************

LCDWriteNibble

btfss	STATUS, C		; Set the register select

bcf	LCD_RS

btfsc	STATUS, C

bsf	LCD_RS

bcf	LCD_RW			; Set write mode

bcf	LCD_D4_DIR		; Set data bits to outputs

bcf	LCD_D5_DIR

bcf	LCD_D6_DIR

bcf	LCD_D7_DIR

NOP				; Small delay

NOP

bsf	LCD_E			; Setup to clock data

btfss	temp_wr, 7			; Set high nibble

bcf	LCD_D7

btfsc	temp_wr, 7

bsf	LCD_D7

btfss	temp_wr, 6

bcf	LCD_D6

btfsc	temp_wr, 6

bsf	LCD_D6

btfss	temp_wr, 5

bcf	LCD_D5

btfsc	temp_wr, 5

bsf	LCD_D5

btfss	temp_wr, 4

bcf	LCD_D4

btfsc	temp_wr, 4

bsf	LCD_D4

NOP

NOP

bcf	LCD_E			; Send the data

return

; *******************************************************************

; *******************************************************************

LCDWrite

;	rcall	LCDBusy

rcall	LCDWriteNibble

swapf	temp_wr,F

rcall	LCDWriteNibble

swapf	temp_wr,F

return

GLOBAL	LCDWrite

; *******************************************************************

LCDRead

bsf	LCD_D4_DIR		; Set data bits to inputs

bsf	LCD_D5_DIR

bsf	LCD_D6_DIR

bsf	LCD_D7_DIR

btfss	STATUS, C		; Set the register select

bcf	LCD_RS

btfsc	STATUS, C

bsf	LCD_RS

bsf	LCD_RW			;Read = 1

NOP

NOP

bsf	LCD_E			; Setup to clock data

NOP

NOP

NOP

NOP

btfss	LCD_D7			; Get high nibble

bcf	temp_rd, 7

btfsc	LCD_D7

bsf	temp_rd, 7

btfss	LCD_D6

bcf	temp_rd, 6

btfsc	LCD_D6

bsf	temp_rd, 6

btfss	LCD_D5

bcf	temp_rd, 5

btfsc	LCD_D5

bsf	temp_rd, 5

btfss	LCD_D4

bcf	temp_rd, 4

btfsc	LCD_D4

bsf	temp_rd, 4

bcf	LCD_E			; Finished reading the data

NOP

NOP

NOP

NOP

NOP

NOP

NOP

NOP

bsf	LCD_E			; Setup to clock data

NOP

NOP

btfss	LCD_D7			; Get low nibble

bcf	temp_rd, 3

btfsc	LCD_D7

bsf	temp_rd, 3

btfss	LCD_D6

bcf	temp_rd, 2

btfsc	LCD_D6

bsf	temp_rd, 2

btfss	LCD_D5

bcf	temp_rd, 1

btfsc	LCD_D5

bsf	temp_rd, 1

btfss	LCD_D4

bcf	temp_rd, 0

btfsc	LCD_D4

bsf	temp_rd, 0

bcf	LCD_E			; Finished reading the data

FinRd

return

; *******************************************************************

; *******************************************************************

LCDBusy

; Check BF

rlcd	LCD_INS

btfsc	temp_rd, 7

bra	LCDBusy

return

GLOBAL	LCDBusy

; *******************************************************************

; *******************************************************************

DelayXCycles

decfsz	delay,F

bra	DelayXCycles

return

; *******************************************************************

END

Appendix 7.3.3  Math.asm

;************************************************************************

;*	Microchip Technology Inc. 2002

;*	Assembler version: 2.0000

;*	Filename:

;*		p18math.asm (main routine)

;*      Designed to run at 4MHz

;* 	PICDEM 2 PLUS DEMO code

;************************************************************************

list	p=18f452

#include	p18f452.inc

#define	_C	STATUS,0

MATH_VAR	UDATA

AARGB0		RES 1

AARGB1		RES 1

AARGB5		RES 1

BARGB0		RES 1

BARGB1		RES 1

REMB0		RES 1

REMB1		RES 1

TEMP		RES 1

LOOPCOUNT	RES 1

GLOBAL	AARGB0, AARGB1, BARGB0, BARGB1, REMB0, AARGB5, REMB1, TEMP

PROG2	CODE

;---------------- 8 * 8 UNSIGNED MULTIPLY -----------------------

;       Max Timing:     3+12+6*8+7 = 70 clks

;       Min Timing:     3+7*6+5+3 = 53 clks

;       PM: 19            DM: 4

UMUL0808L

CLRF    AARGB1

MOVLW   0x08

MOVWF   LOOPCOUNT

MOVF    AARGB0,W

LOOPUM0808A

RRCF     BARGB0, F

BTFSC   _C

bra    LUM0808NAP

DECFSZ  LOOPCOUNT, F

bra    LOOPUM0808A

CLRF    AARGB0

RETLW   0x00

LUM0808NAP

BCF     _C

bra    LUM0808NA

LOOPUM0808

RRCF             BARGB0, F

BTFSC   _C

ADDWF   AARGB0, F

LUM0808NA       RRCF    AARGB0, F

RRCF    AARGB1, F

DECFSZ          LOOPCOUNT, F

bra            LOOPUM0808

return

GLOBAL	UMUL0808L

;----------------  16/8 UNSIGNED DIVIDE	  ------------------------

;       Max Timing: 2+7*12+11+3+7*24+23 = 291 clks

;       Min Timing: 2+7*11+10+3+7*17+16 = 227 clks

;       PM: 39                                  DM: 7

UDIV1608L

GLOBAL		UDIV1608L

CLRF            REMB0			;clears contents of register REMB0

MOVLW           8			;moves 8 into register LOOPCOUNT

MOVWF           LOOPCOUNT

LOOPU1608A      RLCF             AARGB0,W		;contents of reg. AARGB0 rotated one bit to left through carry flag (result in W)

RLCF             REMB0, F		;contents of reg. REMB0 rotated one bit to left through carry flag

MOVF            BARGB0,W		;moves contents of BARGB0 to reg. W

SUBWF           REMB0, F

BTFSC           _C

bra            UOK68A

ADDWF           REMB0, F

BCF             _C

UOK68A          RLCF             AARGB0, F

DECFSZ          LOOPCOUNT, F

bra            LOOPU1608A

CLRF            TEMP

MOVLW           8

MOVWF           LOOPCOUNT

LOOPU1608B      RLCF             AARGB1,W

RLCF             REMB0, F

RLCF             TEMP, F

MOVF            BARGB0,W

SUBWF           REMB0, F

CLRF            AARGB5

CLRW

BTFSS           _C

INCFSZ          AARGB5,W

SUBWF           TEMP, F

BTFSC           _C

bra            UOK68B

MOVF            BARGB0,W

ADDWF           REMB0, F

CLRF            AARGB5

CLRW

BTFSC           _C

INCFSZ          AARGB5,W

ADDWF           TEMP, F

BCF             _C

UOK68B          RLCF             AARGB1, F

DECFSZ          LOOPCOUNT, F

bra            LOOPU1608B

return

GLOBAL	UDIV1608L

end

Appendix 7.3.4  p2plsp18.lkr

// Sample linker command file for 18F452i used with MPLAB ICD 2

// $Id: 18f452i.lkr,v 1.1 2002/02/26 16:55:21 sealep Exp $

LIBPATH  .

CODEPAGE   NAME=vectors  START=0x0        END=0x29       PROTECTED

CODEPAGE   NAME=page     START=0x2A       END=0x7DBF

CODEPAGE   NAME=debug	 START=0x7DC0     END=0X7FFF     PROTECTED

CODEPAGE   NAME=idlocs   START=0x200000   END=0x200007   PROTECTED

CODEPAGE   NAME=config   START=0x300000   END=0x30000D   PROTECTED

CODEPAGE   NAME=devid    START=0x3FFFFE   END=0x3FFFFF   PROTECTED

CODEPAGE   NAME=eedata   START=0xF00000   END=0xF000FF   PROTECTED

ACCESSBANK  NAME=accessram  START=0x0      END=0x7F

DATABANK    NAME=gpr0       START=0x80     END=0xFF

DATABANK    NAME=gpr1       START=0x100    END=0x1FF

DATABANK    NAME=gpr2       START=0x200    END=0x2FF

DATABANK    NAME=gpr3       START=0x300    END=0x3FF

DATABANK    NAME=gpr4       START=0x400    END=0x4FF

DATABANK    NAME=gpr5       START=0x500    END=0x5F3

DATABANK    NAME=dbgspr     START=0x5F4    END=0x5FF    PROTECTED

ACCESSBANK  NAME=accesssfr  START=0xF80    END=0xFFF    PROTECTED

SECTION     NAME=STARTUP    ROM=vectors

SECTION     NAME=PROG1      ROM=page

Appendix 7.4  Datasheets

Appendix 7.4.1  Pic Microcontroler

Appendix 7.4.2  DC/DC Converter PT4122A

Appendix 7.4.3  DC/DC Converter TPS6734IP

Appendix 7.4.4  PWM TL598CN

Appendix 7.4.4  PWM TL598CN

Appendix 7.4.5  Diode 16CTU04S

Appendix 7.4.6  LTC DAC 1451CM8

Appendix 7.4.7  MOSFET IXFX90N20Q

Appendix 7.4.8  MOSFET Driver MAX4420CPA

8. References

BP Solar.  2002.  BP Solar International LLC.  March 2003 <http://www.bpsolar.com/>

“International Ultrafast Rectifier 16CTU04S Data Sheet.”  International Rectifier, 2001.

“IXYS HiPerFET Power MOSFETs IXFX 90N20Q Data Sheet.”  IXYS, 2002.

“Linear Technology Micropower DACs.”  Linear Technology Corporation, 1995. 
“PIC18FXX8 Data Sheet.”  Microchip Technology Inc., 2002.

”Maxim High-Speed, 6A Single MOSFET Drivers Data Sheet.” Maxim Integrated

Products, 1992.

“MPASM and MPLINK PICmicro Quick Reference Guide.”  Microchip Technology Inc.,

2000.

Predko, Myke.  Programming and Customizing PICmicro Microcontrollers.  New York:

McMraw-Hill Companies, Inc., 2002.

“PT4120 Series Data Sheet.”  Texas Instruments Incorporated, 2002.

The Bit Bucket: Maximim Power Point Trackers.  1998.  Team PrISUm.  January 2003.

<http://www.drgw.net/workshop/MPPT/mppt.html>

“TL598 Pulse-Width-Modulation Control Circuits.”  Texas Instruments Incorporated,

1999.

“TPS6734I Fixed 12-v 120-mA Boost-Converter Supply Data Sheet.”  Texas

Instruments Incorporated, 1999.

9. Acknowledgements

We would like to thank our project advisor, Professor Karen Panetta, for her support throughout the year and for giving us the opportunity to work on a challenging and unique project with an amazing team of engineers.

We would also like to thank our project consultants, Matthew Heller, Richard Colombo, and Michael Quaglia, for their generous time, patience, and guidance with the MPPT design.  We have learned valuable engineering project skills that we can apply to future endeavors.

Many thanks also go out to Project Manager Larisa Schelkin, Professor Steven Morrison, George Preble, and Warren Gagosian for their undying willingness to help with any aspect of the project.